IoT-AtHome

System Design & Requirements

A protocol-agnostic, ML-enhanced, voice-enabled home automation platform with a hardened plugin runtime.

Draft  |  2026-04-20

# 1. Overview

IoT-AtHome is a home automation platform designed from a security-first, local-first posture. The trust model drives the architecture, not the other way around. The platform unifies all common home-automation protocols from sub-GHz radio up through Wi-Fi, Matter, and future standards; provides an ML subsystem for anomaly detection, disaggregation, and suggestions; ships a local voice assistant; and supports third-party extension via a signed, capability-scoped plugin runtime.

The design treats every non-core component as untrusted by default. Plugins, devices, and even human sessions operate under capability-scoped permissions. The platform is fully operational without internet access; cloud services are strictly optional enhancements.

## Design principles

- Security before anything. No default passwords, mTLS internal, signed artifacts, capability sandbox.

- Local-first. Full operation offline. Cloud is opt-in, pinned, and revocable.

- Protocol-agnostic core. The core never speaks a protocol directly; protocols live behind plugin boundaries.

- Dumb edges, smart hub. Edge devices measure and act; analytics, ML, and decisions live on the hub.

- Models predict, automations decide. ML outputs are signals, not actuations; humans or explicit rules authorize control.

- Plugin composition over core features. New features land as signed plugins, not as core forks.

# 2. Requirements

## 2.1 Functional

- Ingest and control across sub-GHz RF (433/868/915 MHz), Zigbee 3.0, Z-Wave, Thread/Matter, BLE and BT mesh, Wi-Fi (mDNS, SSDP, MQTT, HTTP/CoAP), IR, KNX, Modbus, with explicit room for new radios.

- Automation engine combining declarative rules with ML-driven suggestions and anomaly detection.

- Voice assistant pipeline: wake word, STT, intent routing, action dispatch, TTS, local by default.

- Plugin SDK allowing third parties to add devices, protocols, ML models, and UI tiles without forking core.

- UX surfaces: web dashboard, mobile app, local API, voice, and a dedicated Command Central wall-panel.

## 2.2 Non-functional

| Attribute | Target |
| --- | --- |
| Security | Zero-trust internal bus, signed plugins, mTLS everywhere, per-plugin capabilities, SLSA-attested builds. |
| Availability | 99.9% of scheduled automations fire; hub survives plugin crashes; graceful offline behavior. |
| Latency | <150 ms local automation trigger to actuation; <800 ms voice intent to action. |
| Scale | 500 devices, 50 plugins, 100 concurrent sessions on a mid-tier SBC (Pi 5 / N100 class). |
| Compliance | ETSI EN 303 645, ISO 27001 / SOC 2 aligned controls, GDPR, OWASP ASVS L2, Matter certification. |
| Privacy | Telemetry off by default; granular opt-in; one-click user-data erase including backups and derived features. |

## 2.3 Constraints

- Runs on Linux (primary), with optional containerized K3s deployment.

- Target build team is small; bias toward boring, well-supported technology.

- Must support 24/7 unattended operation.

# 3. High-Level Architecture

The platform is a tiered architecture: client surfaces on top, an API gateway enforcing authN/Z and TLS, a set of core services communicating over a mutually-authenticated event bus (NATS JetStream), and a plugin host that runs protocol adapters, integrations, and third-party code in sandboxed runtimes.

## Component diagram

+-------------------------------------------------------------+

|  CLIENTS                                                    |

|  Web UI | Mobile | Voice | 3rd-party | CLI | Command Central|

+----+-----------------+-------------------------+------------+

| HTTPS/WSS        | gRPC/TLS                | WSS + mTLS

v                  v                          v

+-------------------------------------------------------------+

|  API GATEWAY   (OAuth2/OIDC, mTLS, rate limit, audit, WAF)  |

+----+--------------------------------------------------------+

|

v

+-------------------------------------------------------------+

|  CORE SERVICES                                              |

|  Device Registry | Automation Engine | Voice Orchestrator   |

|         \              |                    /                |

|          v             v                   v                 |

|   +-------------------------------------------+              |

|   |  EVENT BUS (NATS JetStream, mTLS)         |              |

|   |  device.*  automation.*  ml.*  sys.*      |              |

|   +-------------------------------------------+              |

|          ^             ^                   ^                 |

|          |             |                   |                 |

|  ML Service        Plugin Host        Audit / OTel / SIEM   |

+----------------------+--------------------------------------+

|

+------------------+-+------------------+---------------+

v                  v                   v               v

Zigbee adapter  Z-Wave adapter  433 SDR adapter  Cloud bridges

(plugin)        (plugin)        (plugin)         (Matter, APIs)

## Component responsibilities

| Component | Role | Tech (proposed) |
| --- | --- | --- |
| API Gateway | AuthN/Z, TLS termination, rate limit, audit | Envoy + OIDC (Keycloak/Authelia) |
| Device Registry | Canonical device model, state, capabilities | Rust/Go service + SQLite/Postgres |
| Automation Engine | Rule DAG, scheduler, idempotent executor | Rust/Go; CEL for expressions |
| Event Bus | Pub/sub, durable streams, replay | NATS JetStream (mTLS, per-plugin accounts) |
| Plugin Host | Sandboxed runtime, capability enforcement | WASM (Wasmtime) primary; OCI-container fallback |
| Voice Orchestrator | Wake -> STT -> NLU -> action -> TTS | openWakeWord, Whisper/Vosk, local LLM, Piper |
| ML Service | Feature store, inference, anomaly detection | Python service, ONNX Runtime |
| Observability | Logs, metrics, traces, audit | OpenTelemetry -> Loki/Prometheus/Tempo |

Language split: core in Rust for memory safety, performance, and WASM host maturity. Plugins polyglot via the WASM Component Model. ML in Python, isolated behind a gRPC boundary.

# 4. Deep Dive

## 4.1 Device model (canonical)

One registry, many adapters. Adapters translate protocol frames into canonical entity events on the bus. Matter is treated as the schema baseline.

{

"id": "dev_01HXX...",              // ULID, stable

"integration": "zigbee",           // plugin id

"external_id": "0x00158d0001abcdef",

"manufacturer": "Aqara",

"model": "WSDCGQ11LM",

"capabilities": ["temperature", "humidity", "battery"],

"entities": [

{ "id": "...", "type": "sensor.temperature", "unit": "C", "rw": "r" }

],

"rooms": ["living_room"],

"trust_level": "verified | user_added | discovered",

"last_seen": "2026-04-20T..."

}

## 4.2 Automation engine

- Rule format: declarative YAML/JSON compiled to a DAG. Triggers -> conditions -> actions.

- Expression language: CEL (sandboxed, typed, non-Turing-complete). Embedded Lua/Python is out of scope to limit attack surface.

- Executor: idempotency keys per trigger firing; retriable actions with exponential backoff; dead-letter on permanent failure.

- ML hook: rules may consume ml.<model>.prediction events (e.g. occupancy likely) as triggers or conditions.

## 4.3 Plugin system

This is the most consequential subsystem. Every extension - protocols, integrations, ML models, UI tiles, firmware - lands as a signed plugin with explicit, user-approved capabilities.

### Runtime choice

WASM (Wasmtime) is the default runtime. Faster cold start (<10 ms), deterministic capability sandbox, no kernel syscall surface, smaller footprint than containers. OCI containers remain available for plugins that require Linux drivers (SDR, serial, USB).

### Manifest (signed)

id: zigbee2mqtt-adapter

version: 1.4.2

entrypoint: plugin.wasm

signatures:

- keyid: ...

sig: ...                        # Sigstore / cosign

capabilities:                       # least-privilege, user-approved

bus.publish: ["device.zigbee.*"]

bus.subscribe: ["cmd.zigbee.*"]

net.outbound: ["mqtt://localhost:1883"]

fs.read: ["/etc/iotathome/zigbee/"]

serial: ["/dev/ttyUSB0"]          # elevated: requires user approval

resources:

memory_mb_max: 128

cpu_pct_max: 25

### Isolation guarantees

- Capability-based: no ambient authority. A plugin sees only the bus subjects, files, and hardware it declared.

- Signed and pinned: cosign verification at install; rejects on signature mismatch.

- SBOM required: CycloneDX SBOM bundled; CVE scan at install and on schedule.

- Resource limits: cgroups v2 for container plugins; Wasmtime fuel/memory limits for WASM.

- Bus ACLs: NATS accounts per plugin; core mediates cross-plugin calls.

### SDK languages

Rust, Go, TypeScript, Python (compiled to WASM). One schema (Protobuf) defines the host ABI.

## 4.4 Protocols & adapters

| Band / Standard | Stack | Hardware |
| --- | --- | --- |
| 433/868/915 MHz | rtl_433 + SDR adapter, or CC1101 module | RTL-SDR v4, CC1101 |
| Zigbee 3.0 | zigbee-herdsman (or Rust equivalent) | Sonoff Dongle-E, ConBee III |
| Z-Wave | Z-Wave JS (containerized plugin) | Aeotec Z-Stick 7 |
| Thread / Matter | OpenThread Border Router + chip-tool | nRF52840 dongle |
| BLE / Bluetooth Mesh | BlueZ via D-Bus | Built-in or USB BT 5.x |
| Wi-Fi LAN | mDNS, SSDP, MQTT, HTTP/CoAP discovery | n/a |
| MQTT 3.1.1 / 5.0 | Embedded Mosquitto/EMQX + client plugin | n/a (broker is local) |
| IR | LIRC / Broadlink | USB-UIRT, Broadlink Pro |
| Industrial | KNX/IP, Modbus TCP/RTU, BACnet | Gateways |
| Future | Adapter template; new bands = new plugin | e.g. LoRa/LoRaWAN, UWB |

Design rule: the core never speaks a protocol directly. Protocols live behind the plugin boundary - that is what makes 'all protocols, including future ones' actually maintainable.

## 4.5 Voice assistant

Pipeline: openWakeWord (local) -> Whisper-small or Vosk (local STT) -> intent router (local NLU for closed-domain: lights, scenes, sensors) -> fallback to local LLM (Llama-3.1-8B-Instruct Q4) for open-domain -> action dispatched as a normal automation trigger -> Piper TTS.

- Wake word and STT run locally. Cloud STT is opt-in, per-user, with explicit consent and audit trail.

- Voice data is ephemeral (RAM only) unless user opts into training corpus. If enabled: encrypted at rest, user-owned key.

- Barge-in and continuous-listen are off by default.

## 4.6 ML service

Shipped use cases:

- Anomaly detection - per-device time-series (Isolation Forest or autoencoder).

- Occupancy prediction - room-level, multi-sensor fusion.

- Energy forecasting - next 24 hours per circuit.

- Automation suggestions - frequent-pattern mining on event logs; suggested rules require human approval before activation.

Serving: ONNX Runtime. Models come from a signed model registry (same cosign pipeline as plugins). Models never auto-actuate - they publish predictions; the automation engine decides.

Training: local-first with a federated option. No raw telemetry leaves the house unless the user opts in and the destination is cryptographically pinned.

## 4.7 Data model & storage

| Data | Store | Retention |
| --- | --- | --- |
| Device registry, config | SQLite (small) / Postgres (large) | forever |
| Event stream | NATS JetStream | 30d hot |
| Time-series | TimescaleDB or VictoriaMetrics | 1y default, configurable |
| ML features | Parquet on local disk | per policy |
| Audit log | Append-only, hash-chained (S3/Glacier optional) | 1y min, immutable |
| Secrets | age-encrypted file or Vault | n/a |

# 5. MQTT

MQTT is first-class, with a clear separation between its two roles: south-facing adapter (devices talk to us) and north-facing integration point (external systems talk to us). Both use the same protocol, same broker, different ACL scopes.

## 5.1 Embedded broker

- Ships with the hub. Mosquitto by default; EMQX option for clustering / higher scale.

- Listeners: 1883 disabled by default, 8883 (TLS) and 8884 (mTLS for devices) enabled, 9001 (WSS).

- Zigbee2MQTT, Tasmota, ESPHome, Shelly, Theengs (BLE->MQTT), rtl_433->MQTT all land here via their own adapter plugins.

## 5.2 Topic convention

iot/<plugin_id>/<device_id>/state

iot/<plugin_id>/<device_id>/cmd

iot/<plugin_id>/<device_id>/availability

Plugins may bridge foreign trees (e.g. zigbee2mqtt/#) into their own namespace. Bridging is a declared capability, not ambient.

## 5.3 North-facing integration

- Read-only mirror topic home/events/# for NodeRED, Grafana, scripts, SCADA.

- Command ingress home/cmd/# requires a per-client mTLS cert with topic-scoped ACL.

## 5.4 Plugin capability additions

capabilities:

mqtt.subscribe: ["zigbee2mqtt/#", "tasmota/+/tele/#"]

mqtt.publish:   ["iot/zigbee/+/cmd"]

mqtt.bridge:    ["external-broker://..."]   # elevated

## 5.5 MQTT security

- No anonymous listener. allow_anonymous false is hard-coded; config validator rejects overrides.

- mTLS for devices where firmware supports it; password+TLS is the fallback, not the default.

- Per-client ACLs generated from plugin manifests; wildcard subscribes require explicit user approval.

- Payload size and rate limits per client; stops a compromised sensor from DoS-ing the bus.

- Retained messages are treated as state -> included in the audit log; clearing is a first-class admin action.

- MQTT 5 features used: shared subscriptions for HA plugin pairs, message expiry, reason codes.

- Bridging to external brokers is an elevated capability; requires pinned CA and direction-restricted topics.

# 6. Command Central

A dedicated, always-on control surface: wall-mounted tablet, kitchen touchscreen, or desktop display. Different hardware posture (shared, sometimes public-facing), different threat model, different interaction patterns than the web UI.

## 6.1 Hardware profiles

| Profile | Example | Notes |
| --- | --- | --- |
| Wall panel | 8-13 inch PoE tablet (Fire, Lenovo, custom RPi+DSI) | Primary target. Kiosk mode, auto-wake on motion. |
| Kitchen/hallway display | 10 inch HDMI touch + Pi 4/5 | Shared household device. |
| Dedicated HMI | Industrial panel PC | Garages/workshops; ruggedized. |
| Desktop | Browser on any PC | Same app, windowed. |
| TV dashboard | Chromecast / FireTV cast target | Read-only status wall variant. |

## 6.2 Tech

- PWA-first (React + TypeScript + Tailwind, Vite build) wrapped in a minimal kiosk shell: Electron for x86; WebView + WPE/Cog on RPi; Fully Kiosk Browser for Android tablets.

- Offline-first: IndexedDB cache + service worker; panel still shows last state and queues commands when the core is unreachable.

- Realtime: subscribes to device.*, alert.*, scene.* on the bus via the WebSocket gateway. p95 paint-to-reality under 250 ms.

- No REST polling. Stream or nothing.

## 6.3 Auth model

A wall tablet is physically accessible to anyone in the house, including guests. Treating it as a logged-in admin browser is how home automation systems leak. The panel itself has an identity; the person in front of it authenticates separately and ephemerally.

| Layer | Mechanism |
| --- | --- |
| Device identity | mTLS cert provisioned at enrollment; revocable from core |
| Default session | Household view: read + low-risk actions (lights, scenes, music) |
| Elevated actions | PIN, NFC tag, BLE phone proximity, or voice+PIN - choose per user |
| High-risk actions | Always elevated: unlock doors, disarm alarm, admin settings |
| Session timeout | Elevated session expires on inactivity (default 60s) and on step-away (presence sensor) |
| Admin | Never on panel. Admin is a separate OIDC scope reachable only from the web UI. |

## 6.4 Presence & ambient

- Wake on approach (radar like LD2410, PIR, or BLE RSSI); display off otherwise (privacy + screen life).

- Ambient mode when idle: clock, weather, next calendar event, quiet alerts. No controls visible - prevents shoulder-surfed taps.

- Identity-aware UI (opt-in): surface your scenes and calendar when the hub recognizes you.

## 6.5 Plugin contribution

Tiles are plugin-provided. A plugin ships its own tile components as a signed WASM+JS bundle - same signing chain as backend plugins. Tiles run in a sandboxed iframe with manifest-declared data and action scopes.

ui:

tiles:

- id: solar-live

bundle: tiles/solar.wasm.js

permissions:

data: ["energy.*"]

actions: ["scene.*"]

size: ["2x1", "4x2"]

## 6.6 Security specifics

- CSP: default-src 'self'; connect-src 'self' wss://gateway.local; frame-src 'self' - no third-party anything.

- Certificate pinning to the local CA.

- Screen privacy: sensitive data (PINs, private-room cameras) requires elevated session; ambient never shows them.

- Tamper detection: panel reports app hash + OS integrity to the core; drift = quarantine.

- Audit: every action logged with panel ID + (if elevated) user ID.

- Guest mode: big, obvious toggle that narrows the UI to household view; survives reboot until explicitly disabled.

## 6.7 Offline behavior

- Last-known state stays visible; commands queue locally.

- Safety-critical commands (locks, alarm, garage) do not queue - live confirmation required. Better to fail loud than fire late.

- Reconnect reconciles via CRDT merge; conflicts resolve last-writer-wins with audit trail.

# 7. Water Usage - ESP32-CAM + TFLite Micro

Read a household water meter with a camera and on-device ML, publish canonical consumption events, detect leaks and anomalies via the ML service.

## 7.1 Check for simpler alternatives first

- Pulse output (reed switch / S0) on the meter - use that. 1 pulse = 1 L or 10 L, no CV needed.

- M-Bus / wM-Bus smart meter - use the 868 MHz adapter plugin.

- Legacy mechanical or dial-only - this is where the camera earns its keep.

The integration ships all three variants; users should not do CV when a reed switch exists.

## 7.2 Hardware

| Part | Choice | Why |
| --- | --- | --- |
| MCU + camera | ESP32-S3-CAM (OV2640 or OV5640) | S3 has vector instructions + more PSRAM -> real TFLM perf. |
| Lighting | IR LED ring (850 nm) + IR-pass filter; or white LED on PIR | Meter cupboards are dark; IR avoids visual pollution. |
| Mounting | 3D-printed shroud aligned to meter face | Calibration stability is the #1 accuracy factor. |
| Power | 5 V USB or PoE splitter | Batteries kill duty cycle and accuracy. |

## 7.3 On-device pipeline

Capture (QVGA 320x240 grayscale, 1 fps scheduled)

|

v

Preprocess: warp to calibrated ROI, normalize, contrast stretch

|

v

Digit locator: tiny detector OR fixed crops from calibration

|

v

Per-digit classifier: CNN ~80 KB, 10 classes + "rolling"

|

v

Reading assembler: handles wheel-rolling ambiguity, monotonic constraint

|

v

Confidence check -- low --> buffer raw crop + upload to hub

| high

v

Publish MQTT  iot/water/<id>/reading  { m3: 00142.783, conf: 0.97, ts }

Why per-digit, not whole-number regression: digit wheels rotate and mid-roll looks like 'between 3 and 4'. A whole-number regressor hides that. A classifier with a 'rolling' class plus a monotonic post-filter behaves correctly.

## 7.4 Models

| Model | Size | Target | Notes |
| --- | --- | --- | --- |
| digit_classifier_v1.tflite | ~80 KB int8 | Per-digit wheel | 10 digits + rolling class |
| dial_regressor_v1.tflite | ~120 KB int8 | Analog needle dials | Angle regression, 0-360 -> fraction |

Both int8-quantized with representative-dataset calibration. ESP-DL or TFLite Micro runtime - ship TFLM first, add ESP-DL behind a build flag.

## 7.5 Calibration flow

- 1. User mounts the camera and opens the calibration wizard on the panel.

- 2. User drags corners of the meter face -> perspective transform matrix saved.

- 3. User drags rectangles over each digit wheel -> per-digit ROIs saved.

- 4. User enters current meter reading -> offset anchor for monotonic check.

- 5. Hub ships the calibration bundle back to the ESP32, persisted in NVS.

Calibration is a config artifact, not firmware. Users can recalibrate without reflashing.

## 7.6 Training loop

- ESP32 uploads low-confidence crops (<0.9) to the hub - small, opt-in - with the assembler's best-guess label.

- User reviews mislabelled ones in the web UI during the first two weeks (onboarding task: 'label 20 images').

- Hub fine-tunes a per-household model on top of the base, quantizes, signs, OTAs to the ESP32.

- Base model stays in the signed model registry; per-household deltas use the same signing chain.

Privacy: crops never leave the house unless the user opts into public dataset contribution.

## 7.7 Canonical entities

{

"integration": "water-meter-cv",

"entities": [

{ "id": "water.total",        "type": "sensor", "unit": "m3",   "class": "total_increasing" },

{ "id": "water.flow_rate",    "type": "sensor", "unit": "L/min" },

{ "id": "water.reading_conf", "type": "sensor", "unit": "%" },

{ "id": "water.leak_suspect", "type": "binary_sensor" }

]

}

## 7.8 Anomaly detection (ML service)

| Detection | Method | Signal |
| --- | --- | --- |
| Slow leak | Night-hours baseline (2-5 AM), 14-day median | leak_suspect when baseline > 0 for 3 consecutive nights |
| Burst / open tap | Hampel filter on flow_rate | Immediate push alert + optional auto-shutoff scene |
| Monotonic violation | Reading decreased | Low confidence -> re-inference upload |

## 7.9 Security (ESP32 baseline)

- Secure Boot v2 + Flash Encryption enabled at first flash.

- Per-device X.509 cert provisioned at enrollment (QR-code pairing via the panel or phone).

- mTLS to the MQTT broker, pinned to the home CA. No internet egress by default.

- Signed OTA, A/B partitions, rollback on boot-fail. Same cosign chain as core plugins and models.

- SBOM for firmware registered with the hub's CVE watcher.

- Network isolation: enrollment wizard nudges the IoT VLAN.

- No telnet, no serial-over-wifi, no debug backdoors. Debug builds refuse enrollment.

- Crops only leave the device to the hub; 'send any image to cloud' is never a default.

# 8. Mains Power - 3-phase ESP32

3-phase main incoming power monitoring with CT clamps and voltage taps. Combined with the heating plugin, this is the backbone of the platform's energy intelligence: real power, power factor, harmonics, imbalance, NILM disaggregation, and COP.

## 8.1 Hard prerequisite: electrician + regulation

- Installation by a qualified electrician. Not negotiable. Part P (UK), NEN 1010, NF C 15-100, NEC.

- Non-invasive clamps only on the current side - no cutting conductors.

- Fused, isolated voltage taps. Galvanic isolation between HV side and ESP32 (AC-AC transformer or isolated AFE).

- DIN-rail enclosure, IP-rated, placed next to the consumer unit.

- Firmware disables operation until a commissioning signature is present (installer credentials + date).

The plugin UI's first screen is 'call an electrician'. Not decoration - product posture.

## 8.2 Sensor + AFE choice

Do not build this on ESP32's built-in ADC - noisy, non-linear, temperature-drifts. A dedicated metering IC is the right abstraction.

| Option | When |
| --- | --- |
| ATM90E32AS / ATM90E36A (recommended) | Production. 3-phase metering IC, SPI. Computes V_rms, I_rms, P/Q/S, PF, frequency, THD in hardware. Revenue-grade. |
| ADE9000 / ADE7953 | Higher-end alternative, more headroom, higher cost. |
| ESP32 ADC + raw CTs | Prototyping only. Skip for anything claimed as accurate. |

## 8.3 Sensor topology

| Sensor | Purpose | Part |
| --- | --- | --- |
| CT clamp | Current (I) | SCT-013-000 100A (budget); YHDC SCT-006 or Magnelab SCT-0750 (accurate) |
| Voltage tap | Real V for P/PF | Small AC-AC transformer (ZMPT101B or UL-rated step-down) with fused primary |

Plus: neutral CT (4th) strongly recommended - catches phase imbalance and leakage currents. Choose CT rating to match incoming breaker.

## 8.4 What gets measured

Per phase (L1, L2, L3):

V_rms        [V]

I_rms        [A]

P_real       [W]    signed (+import / -export)

Q_reactive   [VAr]

S_apparent   [VA]

PF           [-]

Frequency    [Hz]

THD_v, THD_i [%]

Totals / derived:

P_total       [W]

energy_import [kWh]  monotonic, persisted NVS + hub checkpoint

energy_export [kWh]  for solar / net-metering

imbalance     [%]    from 4th CT

Events:

sag, swell, outage, phase_loss, overcurrent_phase_X

Energy counters must be monotonic and survive reboots. Stored in NVS with wear-leveling + periodic checkpoint to the hub. Never reset without explicit, audited user action.

## 8.5 Pipeline

ATM90E32 (SPI) --> ESP32-S3

|

+- 1 Hz: per-phase V/I/P/Q/S/PF/F

+- 10 Hz: P_total (burst detection)

+- events: sag/swell/outage (immediate)

+- 5 min: harmonics snapshot, energy checkpoint

|

v  mTLS MQTT

iot/power/<id>/phase/L1   {V,I,P,Q,S,PF,F,ts}

iot/power/<id>/phase/L2   ...

iot/power/<id>/phase/L3   ...

iot/power/<id>/totals     {P,E_in,E_out,imbalance,ts}

iot/power/<id>/event      {type, phase, ts, magnitude}

iot/power/<id>/harmonics  {phase, THD_v, THD_i, bins[]}

Edge stays dumb. All ML, disaggregation, alerting - on the hub.

## 8.6 ML / analytics

| Model | What it does | Approach |
| --- | --- | --- |
| NILM (disaggregation) | Identify individual appliances from aggregate | Event-based detector + signature classifier; NILMTK / seq2point; per-household fine-tune |
| Anomaly detection | Something new draws power at 3 AM | Per-hour baselines + Hampel + isolation forest |
| Appliance-failure prediction | Motor bearing wear via harmonic drift | THD + reactive-power trend; long-horizon, low-urgency |
| Solar self-consumption | Shift flexible loads to export windows | Rule-composition suggestions, user approves |
| Demand forecasting | Next-24h total P for tariff optimization | Gradient-boosted regressor on time, weather, occupancy |
| Phase balancing advisor | 'Move your EV charger to L3' | Imbalance integral over 30 days |

## 8.7 Capacity guardian (deterministic)

The automation engine needs to know service capacity and any contractual limits. At rule compile time, any rule turning on a known-large load is checked against declared capacity and a 15-minute forecast. Conflicts surface as design-time warnings.

Runtime: a capacity guardian subscribes to power.total.P at 10 Hz and sheds load in user-configured priority order when trending toward overload. Safety-critical logic is deterministic, not ML-driven.

## 8.8 Security additions (beyond ESP32 baseline)

- Tamper detection: accelerometer logs and alerts on enclosure movement.

- Write-protected commissioning: CT ratio, voltage-tap transform, service capacity, installer signature are set once and require a physical button + hub-side admin action to change.

- No remote firmware rollback to pre-commissioning versions; signed version floor.

- Air-gap default: no outbound internet; hub is its only peer.

# 9. Heating Flow/Return Temps

Two temperature probes on the heating loop: flow (water leaving the boiler / heat pump into the house) and return. Publish T_flow, T_return, delta T. When combined with the power meter plugin, derive COP in real time.

## 9.1 Signals and meaning

| Signal | Meaning |
| --- | --- |
| delta T (flow - return) | Heating system health. Design delta T: ~5-7 K heat pumps, ~15-20 K gas boilers. Drift indicates balance or flow issue. |
| T_flow vs outdoor | Validates weather-compensation curve. Over-spec = wasted energy. |
| delta T + flow rate | Actual thermal kWh delivered. Without flow, delta T is qualitative only. |
| Thermal kWh / Electrical kWh | COP - the number heat pump owners actually care about. |
| Short-cycling pattern | Heat pump cycling too often -> sizing / control issue. |
| Defrost detection | Transient inverted delta T is normal; frequent / long = icing. |

## 9.2 Sensor choice

| Tier | Sensor | Accuracy | Use |
| --- | --- | --- | --- |
| Good enough | DS18B20 (1-Wire, waterproof probe) | +/-0.5 K; ~+/-0.2 K matched pair | Gas boilers at 15-20 K delta T |
| Accurate | PT1000 + MAX31865 (SPI, 15-bit) | +/-0.15 K; +/-0.05 K matched pair | Required for heat pump COP |
| Overkill | Class A PT100 + bridge | Revenue-grade | Skip |

Default to PT1000 matched pair, factory-calibrated together. A 0.5 K error on a 5 K delta T is 10% error on thermal output - wrecks COP.

## 9.3 Mounting

- Strap-on (non-invasive): sensor clamped to copper pipe with thermal paste, covered with insulation sleeve. DIY-friendly. Requires the insulation - un-insulated reads 2-4 K low.

- Immersion pocket (plumber required): probe in thermowell, direct water contact. ~0.1 K better, much more stable.

Calibration at commissioning: both probes in the same bucket of water -> capture offset -> store per-probe correction in NVS. Do this before installing on the pipe. Skipping this is why most DIY heat-pump COP claims are wrong.

## 9.4 Hardware integration

- Option A (preferred): daughter-board on the power-meter ESP32-S3 - 2x MAX31865 over SPI. Same enclosure, commissioning, cert, mTLS session.

- Option B (standalone): separate ESP32-S3 near the heating system. Same firmware family and security baseline.

## 9.5 Pipeline

PT1000 --> MAX31865 --> SPI --> ESP32

PT1000 --> MAX31865 --> SPI ---'

|

+- 1 Hz: T_flow, T_return, delta T

+- event: short-cycle (3 on-off within 10 min)

+- event: defrost (delta T inverted > 60s)

+- 5 min: checkpoint to hub

|

v  mTLS MQTT

iot/heating/<id>/flow    {T, ts}

iot/heating/<id>/return  {T, ts}

iot/heating/<id>/delta   {delta_T, ts}

iot/heating/<id>/event   {type, ts, details}

## 9.6 Canonical entities

{

"integration": "heating-loop",

"entities": [

{ "id": "heating.T_flow",         "type": "sensor", "unit": "C" },

{ "id": "heating.T_return",       "type": "sensor", "unit": "C" },

{ "id": "heating.delta_T",        "type": "sensor", "unit": "K" },

{ "id": "heating.short_cycle",    "type": "binary_sensor" },

{ "id": "heating.defrost_active", "type": "binary_sensor" }

]

}

## 9.7 Optional flow rate -> thermal kWh

- Ultrasonic clamp-on (Grundfos VFS, Sika): non-invasive, +/-2% with good coupling.

- Inline turbine / impeller: plumber, +/-1%.

- Read from heat-pump Modbus: most modern heat pumps expose flow; use the Modbus adapter.

If present: publish heating.flow_rate and heating.thermal_power. Thermal power = flow_rate x rho x c_p x delta T. Glycol mix configured at commissioning.

## 9.8 COP cross-plugin

The ML service subscribes to both heating.thermal_power and power.heat_pump.P, and publishes:

- heating.cop.instant (1 Hz, rolling 60s average)

- heating.cop.daily (kWh_thermal / kWh_electrical, daily)

- heating.cop.seasonal (SCOP rolling 12 months)

The power-meter plugin must know which circuit is the heat pump - a commissioning checkbox, no new glue code. If sub-metering is absent, fallback = total minus baseline (noisy, clearly labelled). This is the architectural test: a cross-plugin feature requires no bespoke integration, only a subscription.

## 9.9 Automation hooks

- trigger: heating.delta_T < 2 K for 10 min AND heating.flow_rate > 0

then: notify "Heating system not transferring heat - check pump or air lock"

- trigger: heating.short_cycle == true

then: notify "Heat pump short-cycling - check sizing or buffer tank"

- trigger: heating.cop.daily < weather_adjusted_expected * 0.85 for 3 days

then: notify "Heat pump efficiency dropped - service recommended"

- trigger: heating.T_return < 2 C

then: activate scene "freeze_protection"

# 10. Security Architecture

## 10.1 Trust model

- Core = TCB. Everything else is untrusted until proven otherwise (signed + capability-scoped).

- User = sovereign. Keys held locally. Cloud is optional.

## 10.2 Controls (ETSI EN 303 645 + OWASP ASVS L2)

| Control | Implementation |
| --- | --- |
| No default passwords | First-boot wizard forces strong credentials + 2FA |
| Secure update | Signed, atomic, A/B partitions, rollback; TUF metadata |
| Vuln disclosure | .well-known/security.txt, public PGP key, 90-day SLA |
| Keep software updated | Auto-update staged rollout; CVE watcher per SBOM |
| Secure communication | mTLS everywhere internal; TLS 1.3 external; HSTS |
| Minimize attack surface | No default open ports; admin on loopback / VPN only |
| Software integrity | Measured boot where HW supports; signed binaries; cosign |
| Personal data protection | Encryption at rest (LUKS + per-table); user-held keys for exports |
| Resilience to outages | Full local operation; cached credentials; offline ML |
| Telemetry | Off by default; granular opt-in; local-only audit always on |
| Delete user data | One-click full erase, including backups and derived features |
| Secure install | Reproducible builds; SLSA L3 target |

## 10.3 Supply chain

- Reproducible builds, SLSA provenance, CycloneDX SBOMs per artifact, cosign signatures, TUF update metadata.

- Plugin marketplace requires: signed author identity, SBOM, automated scan pass, human review for serial / raw-socket / elevated capabilities.

## 10.4 Threat model highlights

- Malicious plugin: contained by capability sandbox + bus ACL + resource limits; worst case is its own subject tree.

- Compromised cloud partner: isolated to bridge plugin; core state untouched; credentials revocable.

- RF spoofing (433 MHz): replay attacks assumed; 433 sensors never used as sole input for security-critical automations. Engine enforces via capability tags.

- Voice injection (ultrasonic/laser): high-impact intents require confirmation; optional PIN.

# 11. APIs

- External: REST + WebSocket (OpenAPI 3.1), OAuth2/OIDC with PKCE, scoped tokens. Optional GraphQL read layer for UI.

- Internal: gRPC over mTLS service-to-service; NATS for events.

- Plugin ABI: Protobuf-defined, versioned, backward-compatible for a full major cycle.

## REST examples

POST /api/v1/automations

GET  /api/v1/devices?room=kitchen

POST /api/v1/devices/{id}/commands   { "entity": "...", "action": "...", "value": ... }

WS   /api/v1/stream?topics=device.*

# 12. Scale & Reliability

- Vertical first. Target is a single home. A Pi 5 with NVMe handles the stated load.

- Horizontal option: core services are stateless behind the registry / DB; HA pair via keepalived or K3s. Zigbee / Z-Wave coordinators are stateful hardware - plugin supports active-passive with takeover.

## Failure domains

- Plugin crash -> supervisor restart with backoff; event bus queues buffer commands.

- Core crash -> systemd restart; state rehydrated from registry + JetStream replay.

- Power loss -> UPS signaling plugin triggers graceful-shutdown automations.

## Observability

Every event carries a trace ID. A failing automation is a debuggable span tree, not a log grep.

# 13. Trade-off Analysis

## 13.1 Core architecture

| Decision | Alternative | Why chosen | What it costs |
| --- | --- | --- | --- |
| WASM plugin runtime (default) | Containers everywhere | Faster, smaller, stronger sandbox | HW-access plugins still need containers -> two runtimes |
| Rust core | Go or Python | Memory safety + perf + WASM host maturity | Smaller contributor pool early |
| NATS JetStream | Kafka / Redis Streams | Lightweight, mTLS accounts, runs on SBC | Smaller ecosystem |
| CEL for rules | Embedded Python / Lua | Sandboxed, typed, no RCE surface | Power users may want loops |
| Local-first ML | Cloud ML | Privacy, offline, no recurring cost | Smaller models, slower training |
| Matter as schema baseline | Custom schema | Future-proof, interop | Some early devices need shims |
| Own voice stack | Alexa / Google | Privacy, offline | Worse open-domain accuracy |

## 13.2 MQTT

| Decision | Alternative | Why | Cost |
| --- | --- | --- | --- |
| MQTT as adapter, not primary bus | MQTT everywhere internally | NATS has better auth / multi-tenant; no retained-msg footguns | Two messaging systems |
| Bundle Mosquitto by default | BYO broker | Works out of the box | Larger install |
| ACLs from plugin manifest | Hand-edit mosquitto.conf | Single source of truth | Manifest schema grows |

## 13.3 Command Central

| Decision | Alternative | Why | Cost |
| --- | --- | --- | --- |
| PWA + kiosk shell | Native apps per platform | One codebase, fast iteration, good offline | Lockdown work per OS |
| Device cert + ephemeral user auth | Shared panel login | Matches physical reality of shared panels | More UX work |
| Tiles in sandboxed iframes | Freeform plugin React | Prevents escalation | Constrained API |
| WebRTC cameras, local signalling | MJPEG / cloud relay | Low latency, no cloud dep | WebRTC setup complexity |

## 13.4 Water meter

| Decision | Alternative | Why | Cost |
| --- | --- | --- | --- |
| ESP32-S3 + TFLM on-device | RTSP camera -> hub inference | Offline, low bandwidth, no video leaves cupboard | Lower model ceiling |
| Per-digit classifier | End-to-end regressor | Handles rolling digits correctly | Calibration required |
| On-device low-conf upload | Always upload | Bandwidth + privacy | Some mislabels never caught |
| Per-household fine-tune | One-size-fits-all | Handles unusual meters | Small training infra on hub |
| IR lighting default | White LED | Works in closed cupboards | IR-pass filter needed |

## 13.5 Mains power

| Decision | Alternative | Why | Cost |
| --- | --- | --- | --- |
| Dedicated metering IC (ATM90E32) | ESP32 ADC + DSP firmware | Revenue-grade out of the box | +$3 BOM, IC lock-in |
| Voltage taps per phase | Assume nominal 230 V | Real P, PF, harmonics, sag/swell | Electrician install |
| Neutral CT (4th) | Skip | Imbalance + leakage detection | +1 CT ~$10 |
| All analytics on hub | Edge ML for disaggregation | Hub has compute + retraining loop | Needs hub reachable; MQTT buffers outages |
| Deterministic capacity guardian | ML-driven load shed | Safety-critical must be predictable | Less 'clever' |
| Tamper accelerometer | Skip | Consumer-unit tamper sensitivity | +$1 BOM |

## 13.6 Heating

| Decision | Alternative | Why | Cost |
| --- | --- | --- | --- |
| PT1000 + MAX31865 default | DS18B20 default | 10x better delta T, required for COP | Higher BOM + SPI |
| Strap-on default mount | Immersion default | DIY install covers 90% of users | ~0.1-0.3 K worse |
| Piggyback on power-meter ESP32 | Always separate | Fewer devices / certs / enclosures | Couples two plugins' hardware |
| Flow sensor optional | Required | delta T alone still diagnostic | COP gated on flow - disclosed |
| COP via ML-service subscription | Bake COP into one plugin | Validates plugin architecture | Depends on circuit labelling |

# 14. Milestones

| ID | Name (duration) | Scope |
| --- | --- | --- |
| M1 | Skeleton (4 wks) | Core bus + registry + API gateway + auth + Zigbee adapter + web UI stub. Security baseline (mTLS, signing, SBOM) from day one. |
| M2 | Plugin SDK (4 wks) | WASM host, capability model, published SDK + 3 adapters (Z-Wave, 433 SDR, Matter). |
| M3 | Automation + Observability (3 wks) | CEL rule engine, OpenTelemetry end-to-end, audit log. |
| M3.5 | Command Central v1 (3 wks) | PWA shell, kiosk wrapper (RPi + Fire tablet), core tiles, device cert enrollment, PIN elevation, offline cache. |
| M4 | ML anomalies + edge-ML reference plugins (4 wks) | First two ML models + approval flow. Water meter, mains power (measurement + capacity guardian), heating flow/return plugins ship as reference integrations. |
| M4b | COP derivation | Cross-plugin ML-service subscription wiring; requires power meter deployed and heat-pump circuit labelled. |
| M5 | Voice (4 wks) | Wake + STT + NLU + TTS; closed-domain intents only. NILM training pipeline and disaggregation tile extension. |
| M6 | Hardening + cert prep (3 wks) | Pen test, ETSI 303 645 checklist, public vuln disclosure program. |

# 15. Revisit-later

## 15.1 Platform

- Multi-home / tenancy: current design is single-home. Federation via CRDT-based sync before users accumulate pain.

- Plugin marketplace: signed + scanned is table stakes; revocation UX and abandoned-plugin policy are the hard parts.

- LLM in the loop: if the local LLM composes automations as an agent, extend the capability model to the LLM - it should not be more privileged than a plugin.

- Regulatory drift: EU Cyber Resilience Act (CRA) takes effect 2027. SBOM + update + vuln-disclosure commitments should already comply; re-audit annually.

- Storage tiering: TimescaleDB on a Pi is fine at 500 devices; at 5000+ the registry / TSDB split becomes real and needs a migration story.

## 15.2 MQTT

- Sparkplug B for industrial users, once the user base asks.

- MQTT over QUIC (EMQX supports it) once clients do; better for mobile / flaky links.

## 15.3 Command Central

- Gesture + gaze control (kitchen/bathroom - wet/greasy hands), via the same radar sensors used for presence.

- Multi-panel handoff: start a timer on the kitchen panel, walk to the living room, see it pick up there. Needs CRDT sync solid.

- E-ink secondary displays for always-on info without light pollution in bedrooms.

## 15.4 Water

- Electric meter + gas meter with the same pipeline - mostly a new training set. Make meter-cv generic, ship three integrations off it.

- Two-camera redundancy for high-value meters (cross-check).

- ESP32-P4 when mature - dedicated camera pipeline, more NN perf.

- Audio-based leak detection (ultrasonic mics on pipes) - separate plugin.

## 15.5 Mains power

- Phase-angle-aware NILM: per-phase disaggregation; materially better, needs more data.

- Dynamic tariff integration (Tibber, Octopus Agile, Nord Pool) as a hub-side plugin publishing 'cheap now' signals.

- Grid-service participation (demand response, V2G) - regulated, opt-in, country-specific.

- Sub-metering (per-circuit CTs) - different plugin, same firmware stack, 6-12 CTs. Makes NILM nearly unnecessary.

- Safety interlock across domains: capacity guardian + EV + solar coordinating via formal plugin-to-plugin negotiation - worth a dedicated design pass once two exist.

## 15.6 Heating

- Per-zone return temps (multi-zone systems): same plugin, more probes, declared at commissioning.

- Refrigerant-circuit sensors (evaporator / condenser) - invasive, service-level installs only.

- Hydraulic separator detection with buffer tank: delta T across primary vs secondary loops differs. Topology selector in commissioning wizard.

- Gas meter correlation: pairing with a gas-meter plugin (same CV meter-reading pattern as water) gives combustion efficiency estimates.